Nudo Products, Inc.										Endurex 300 Series

(Includes Endurex 300, 305, 310, 315, 320, 325, 330, 335 & 350P)

Section 07430   |  Composite Panels

Part 1: General

1.01 Section Includes

Specifier Note: Endurex 300 Series, (300, 305, 310, 315, 320, 325, 330, 535 and 350P). Delete if not required for project.

A.	Laminated panels for use as glazing infill or railing-type channel inserts.

1.02 Related Sections

Specifier Note: Delete any sections below not relevant to this project; add others as required.

A.	Section 05100—Structural Framing.

B.	Section 06100—Back Up Walls.

C.	Section 07210—Building Insulation.

D.	Section 07600—Flashing and Sheet Metal.

E.	Section 07920—Joint Sealants.

F.	Section 08500—Windows

G. Section 08900--Storefront/Curtain Wall

H. Section 09200--Interior Finishes

1.03 References

Specifier Note: Delete references from the list below that are not actually required by the text of the edited section.

A.	ASTM International (ASTM):

1.	ASTM E84 - Surface Burning Characteristics.

Specifier Note: Delete if not required.

2.	ASTM D1781 - Climbing Drum Peel for Adhesives.

Specifier Note: Delete if not required.

3.	ASTM E72 - Strength Tests for Panels for Building Construction.

B. Aluminum Association

1. AA-C22-A41: Anodized Clear

2. AA-C22-A42: Integral Color Anodized

C. American Architectural Manufacturers Association (AAMA)

1. 603.8-92 Pigmented Organic Coatings on Aluminum (Polyester)

2. 605.2-92 High Performance Organic Coatings on Aluminum - (Kynar)

A.	General: Submit listed submittals in accordance with Conditions of the Contract and Division 01300 Submittal Procedures

Section.

B.	Submit manufacturer’s product data, storage, handling and preparation requirements and installation instructions.

C.	Samples: Submit selection and verification samples for finishes, colors and textures. Submit 2 samples of each type of panel,

trim and fastener

1.04 Submittals

Specifier Note: Delete selection samples if colors have already been selected.

D.	Shop Drawings: Submit shop drawings showing layout, profiles and product components, including anchorage, accessories,

finish colors, patterns and textures. Indicate location and dimension of joints and fastener attachment.

E.	Test Reports: Certified test reports showing compliance with specified performance characteristics and physical properties.

1.05 Quality Assurance

A.	Manufacturer Qualifications: Manufacturer shall have a minimum of ten years experience in the manufacture of composite

architectural wall panels.

B.	Installer Qualifications: Installer shall be experienced in performing work of this section and be specialized in the installation of

similar work required on this project.

C.	Pre-Installation Meetings: Conduct pre-installation meetings to verify project requirements, substrate condition, installation

instructions and warranty requirements. Field measurements shall be taken prior to completion of manufacturing and cutting.

Specifier Note: Include a mock-up if the project size and/or quality warrant taking such a precaution.

D. Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

1.06 Delivery, Storage and Handling

A.	Lead Time: Comply with manufacturer’s ordering instructions and lead time requirements to avoid construction delays.

B.	Deliver materials in manufacturer’s original, unopened, undamaged containers with identification labels intact.

C.	Storage and Protection: Stack materials on platforms or pallets, covered with suitable ventilated covering. Store materials 	protected from exposure to harmful environmental conditions and at temperature and humidity conditions recommended by 	the manufacturer.

1.7 Project Conditions

A.	Maintain environmental conditions (temperature, humidity, and ventilation) recommended by manufacturer.  Do not install 	products under environmental conditions outside manufacturer’s absolute limits.

1.8 Warranty

A.	Manufacturer’s Warranty: Submit, for owner’s acceptance, manufacturer’s standard warranty document executed by 	authorized company official. Manufacturer’s warranty is in addition to, and not a limitation of, other rights Owner may have 	under the Contract Documents.

Specifier Note: Endurex 300 Series.

B.	Panel Lamination Warranty: Five years commencing on Date of Substantial Completion.

Specifier Note:  Endurex 300 Series.

C.	Finish Warranty:

Specifier Note: Delete finish warranty not required. Reference Coatings Manufacturer's Finish Warranty as appropriate.

1.	Polyester: Five years.

2.	Acrylic Urethane: Five years.

3.	Kynar 500®: Twenty years.

4.	Kynar 500® Metallic: Twenty years.

5.	Anodized: Twenty years

6. Porcelain - Twenty Years.

Part 2: PRODUCTS

2.1 Manufacturers

A.	Manufacturer: Nudo Products, Inc. 1500 Taylor Avenue, Springfield, IL 62703; Telephone: (800) 826-4132, (217) 528-5636;

Fax: (217) 528-8722; E-mail: info@nudo.com; website:www.nudo.com

Specifier Note: Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

B.	Substitutions:  Not permitted.

C.	Requests for substitutions will be considered in accordance with provisions of Section 01600.

1.	Acceptable Alternatives: Panels of similar composition providing manufacturer has a minimum of twenty years experience.

2.	The materials and products specified in this section establish a standard of required function,

dimension, appearance and quality to be met by any proposed substitution.

3.	No substitution will be considered unless written request for approval has been submitted by the

bidder and has been received by the Architect ten days prior to the date for receipt of bids.

4.	Each request shall include the name of the materials and a complete description of the proposed

material, including test performance and any other information necessary for evaluation.

Specifier Note: Delete if not required.

B.	Product: Endurex 300 Series Architectural Panels as manufactured by Nudo.

1.	Panel Composition:

a.	Face Skin: [.032"] [.040"] [.050"] [.060"] prefinished aluminum painted to match Architect’s

color selection.

b.	Stabilizer

1.	[Hardboard] [.125"] [.250"]

2. 	[Composite] [.120"] [.250"]

3. 	[Corrugated Polypropylene] [.158"] [.236"] [.315"] [.394"]

4.	[Fiber Cement] [.125"] [.158"] [.236"] [.315"]

[.394"] [.472"]

5.	[Fire-Rated Gypsum] [.500"] [.625"]

6.	[Plywood] [.250"] [.500"] [.750"] [1.00"] [1.125"]

Specifier Note: Delete back skin not required.

c. Core: Sizes below used in standard 1" overall panel.  Additional core thickness available for

panels up to 4" overall thickness.

1. 5/8" Expanded Polystyrene (EPS) Foam

2. 5/8" Extruded Polystyrene (Closed Cell) Foam

3. 5/8" Polyisocyanurate (ISO) Foam

c.	Back Skin: [.032"] [.040"] [.050"] [.060"] prefinished aluminum to match Architect’s

color selection.

2. Panel Tolerances:

a.	Thickness: 1/32 inch (0.8 mm).

b.	Length and Width: +0, -1/16 inch (1.6 mm).

c.	Squareness: 1/64 inch per lineal foot (1.3 mm/m).

Specifier Note: Delete attachment system not required. Attachment systems are provided by manufacturer's other than Nudo.

3.	Attachment System: Two-part extruded aluminum moldings.

4.	Attachment System: Two-part extruded aluminum moldings.

5.	Attachment System: One-part roll formed aluminum moldings.

2.3 Accessories

Specifier Note: Delete if not required.

A.	Recommended for use as an infill panel component in window and curtain wall systems. Related material to complete

installation as recommended by the manufacturer.

B.	Fasteners and moldings as required for panel system’s design by system manufacturer. Fasteners shall be coated or stainless

steel.

C.	Sealants: Protects against moisture intrusion as recommended by the sealant manufacturer. Sealants with a 20 year life are

recommended.

2.4 Finish

A.	Exposed finish:

1.	Polyester

2.	Kynar® 500

3.	Kynar® 500 Metallic

4.	Anodized

5. Porcelain

PART 3: EXECUTION

3.1 Examination

A. Verify substrate conditions prior to installation to determine if they have been properly prepared. Verify that site conditions are 	acceptable for infill or glazing panel systems.

B.	Do not proceed with work until unsatisfactory conditions have been corrected in a manner acceptable to installer. Starting 	work within a particular area will be construed as installer's acceptance of surface conditions.

C. Maximum deviation from vertical and horizontal alignment of substrate shall be no more than 1/4 inch in 20 feet (1 mm in 1 m).

3.2 Preparation

A.	Clean surfaces thoroughly prior to installation.

B.	Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under 	the project conditions.

C.	Comply with manufacturer’s product data including product technical bulletins, product catalog installation instructions, and 	product carton instructions.

D.	Surfaces to receive panels shall be even, smooth, sound, clean, and free from defects detrimental to panel installation.

E.	Field measure and verify dimensions as required.

F.	Protect adjacent areas or surfaces from damage as a result of the Work of this Section.

3.3 Installation

A.	Install in accordance with manufacturer’s instructions.

B.	Sheathing and water resistant membrane (if specified) by others.

C.	Erect panels level and true to intended plane.

D.	Maximum deviation from vertical and horizontal alignment of erected panels shall be no more than 1/4 inch in 20 feet 
(1 mm in 1 m).

E.	Conform to panel manufacturer’s instructions for attachment systems.

F.	Weather seal all joints as required using methods and materials as recommended by the panel manufacturer.

3.4 Protection

A.	Remove temporary coverings and protection to adjacent work areas. Repair or replace damaged installed products. Clean 	installed products in accordance with manufacturer’s instructions prior to Owner’s acceptance.

B.	Remove and legally dispose of construction debris from project site.

C.	Protect installed products until completion of project.

D.	Touch-up, repair or replace damaged products before Substantial Completion.

— END OF SECTION —